19 September 2017 Doc\book\errata 3rd and 4th printings

.

ERRATA AND SUPPLEMENTS

for the third and fourth printings of

Doppler Radar and Weather Observations, Second Edition-1993

Richard J. Doviak and Dusan S. Zrnić

Academic Press Inc., San Diego, 562 pp.

ISBN 0-12-221422-6.

This errata also applies to the copies of the second edition, 1st and 2nd printings, published in 2006 by

Dover Publications, Inc., Mineola, New York.

International Standard Book Number: 0-486-45060-0

Page  Para.    Line	Remarks:  Paragraph 0 is any paragraph started on a previous page that carries over to the current page. Section heading are not counted as a line, but equations are so counted. A sequence of dots is used to indicate a logical continuation to existing words in the textbook (e.g., see errata for p.14; or that for Fig. 3.3 caption; etc.)

xxi		θ	modify definition to read: “is the zenith angle (Fig. 3.1); also the angle from the axis of a circularly symmetric beam (p. 34); also potential energy

14	2	2	change to read: “…index n = c/v with height (or, because the relative permeability μr of air is unity, on the change of relative permittivity, εr = ε/εo = n2, with height).

15	1	7	insert the reference (Born and Wolf, 1964, p. 87)

17	1	2-6	line 2, change “T=300 K” to “T=290 K”; line 4, change this equation to read: N ≈ 0.268×(103 + 1.66×102) ≈ 312; and line 6 change “1.000300” to “1.000312”.

23	Eq. (2.29)	change n(h) to N(h) and ns to Ns

0	10	change “refractive index” to “refractivity N”

30	2	9	replace the italicized “o” from the first entry of the word “oscillator” with a regular “o”, but italicize the  “o” in the second entry of the word “oscillator”

3	7	delete the parenthetical phrase

34	Eq. (3.2)	replace D with Da

2	3	change “intensity” to “power density”

6	change to read: “defines the maximum directivity of the transmitting antenna.”

35	1	9	at the end of the last sentence add: with origin at the scatterer.

2	10	the equation on this line should read:

Eq. (3.6)	and on the line after this equation, change “Km” to “Kw”

36	0	7	delete “|Km|2 ≡”

9	change the end of this line to read: “Ice water has a |Kw|2 ≡”

Fig.3.3	caption		revise to read: “…..(a) Liquid water; the square of the complex refractive index m2 (at 0o C) is…. “

Furthermore, the sign of j everywhere in this caption needs to be changed from + to -.

40	Eq. (3.14b)	replace subscript “m” with “w”

46	1	7	change to read: “…, and that g accounts for losses in the antenna, the radome, and in the transmission lines from the antenna to the point where Pt and Pr are measured.

47	Table 3.1	1) change title to read: “The next generation radar, NEXRAD (WSR-88D), Specifications”

2) change “Beam width” to “Beamwidth”

3) change footnote b to read: “Initially the first several radars transmitted circularly polarized waves, but now all transmit linearly polarized waves”. 4) change footnote c to read: “Transmitted power, antenna gain (including radome loss), and receiver noise power are referenced to the antenna port.

48	0	4-5	change 2x10-7 to 4.2x10-7, and 6.3 to 7.3.

61	Eq. (3.40b)	place  before va

0	14	last line change to “velocity limits (Chapter 7).”

68	3	7-8	because the overbar (i.e., ) is used in later chapters to denote spatial averages weighted by the antenna pattern and range resolution functions, change to read as: “…is thus the expected power E[P(τs)].”

4	1	start this sentence as “E[P(τs)] does not change…”

69	0	6, 10	change to E[P(τs)].

71 	Eqs. (4.4a, b)	insert (1/ ) in front of the sum sign in each of these equations

3	6	replace “page 418” with “page 498”.

Eq. (4.6)	delete the first ‘2’.

72	0	4	change to: “.... and a mean or expected value E[P(τs)] =.”

2	1	change to E[P(τs)]

3	remove footnote and its symbol appended to E[Pi]

73	Eq. (4.11)	change “” to “”.

Eqs. (4.12), (4.14), (4.16):   change “” to “”.

Eqs. (4.14), (4.16)	change to , and change to

75	1	6	change to “G(0) =1”

2	16	change (4.12) to (4.14)

18	change “” to “”.

76   Fig.4.5 	change second sentence in caption to read:  “The broad arrow indicates sliding of....”

77	0	12	change “mean” to “expected”

13	change “” to “”

Eq. (4.21)	change “” to “”

Eq. (4.22)	delete  |W(r)|2

78	Fig. 4.7	change the argument ‘r’ in |W(r)|2 to ‘’

79	2	3	change to: “…a scatterer at r has the approximate range-dependent…”

8-9	change to: “…the weighting function about its peak at any range r = r0.”

82	Eq. (4.34)	change “” to “”.

Eq. (4.35)	change “” to “E[P(mw)]”

1	9	should read: “.. is the reflectivity factor of spheres.”

Eq. (4.38)	subscript “τ” should be the same size as in Eq.(4.37).

84	Eqs. (4.39), (4.43)	change “” to “”.

85	0	4	change “” to “”.

Problem 4.1	change “” to “E[P]” in two places.

Eq. (5.40)	change the arguments and r1 of Ws to and r1, and change to

108	1	1	change “stationary” to “steady”

1	11	change “” to “E[dP]”.

Eq. (5.42)	change “” to “E[dP(v)]

15	change “” to””

Eq. (5.43)	change “” to””

1		after Eq. (5.43) insert “is the differential power from all the elemental volumes having the Doppler velocity v centered in the interval dv.”

3	2-3	change to read: “…..by new ones having different spatial configurations, the estimatesof …”

109	Eq. (5.45)	change “” to “(r0)]”

change footnote “4” to read: “The overbar on a variable denotes a spatial (i.e., volume) average, whether or not the average is weighted.”

113	1	1-4	change to read: “Assume scatterer velocity is the sum of steady vs(r)and turbulent vt(r,t) wind components. Each contributes to the width of the power spectrum (even uniform wind contributes to the width because radial velocitiesvs(r)vary across V6; steady wind also brings new....”

2	10	delete the sentences beginning on line 10 in paragraph 2 with “Furthermore, we assume...” and ending in paragraph 3, line 3 with “...scatterer’s axis of symmetry).”

114	0	5	insert and modify after “….initial phases”: Here expectations are made over an ensemble of scatterer configurations (see supplement to Section 10.2.2). Because R(0) is proportional to the expected power E[P(r0)], and because

(5.59c)

[i.e., from Eq. (4.11)], where  is the expected backscattering cross section  (expectations computed over the ensemble of ) of the kth hydrometeor, it follows that is proportional to and …..”

2	2-4	modify to read: “...mechanisms in Eq. (5.59b) act through product terms.  Furthermore, the kth scatterer’s radial velocity vk can be expressed as the sum of the velocities due to steady and turbulent winds that move the scatterer from one range position...”

6-9	delete these lines and replace with:

“…Eq. (5.59a), the velocities vs(r) and vt(r, t) associated with steady and turbulent winds can each be placed into separate exponential functions that multiply one another.  Thus the expectation of the product can be expressed by the product of the exponential containing vs(r) and the expectation of the exponential function containing vt(r, t); these exponential functions are correlation functions. The Fourier transform of R(mTs), giving the composite spectrum S(f), can then be expressed as a convolution of the spectra associated with each of the three correlation functions.  There are other de-correlating mechanisms (e.g., differential terminal velocities, antenna motion, etc.,) that increase the number of correlation functions and spectra to be convolved.  It is shown that, ….”

115	3	1	“R” in “Rk” should be italicized to read “Rk”

7	change “Eq. (5.59b)” to “Eq. (5.59a)”

14	change these lines and Eqs. (5.64) to read: “Because the correlation coefficient can be related to the normalized power spectrum Sn(f) by using Eq. (5.19), and because the Doppler shift f = -2v/λ, ρ(mTs) can be expressed as

(5.64)

116	0	1-4	change these lines to read: where is the estimated normalized power spectrum in the frequency domain,  is the estimated normalized power spectrum in the Doppler velocity domain, and these two power spectra are related as

.				(5.65)

By equating Eq. (5.63) to Eq. (5.64), and assuming all power is confined within the Nyquist limits,, it can be concluded that

,				(5.66)

1	1-7	change to read: “where the expectation Ev is taken over the ensemble of velocity fields (for additional explanation of Ev see supplement for Section 10.2.2). Thus, for homogeneous turbulence, at least homogeneous throughout the resolution volume V6, the expected normalized power spectrum is equal to the velocity probability distribution.  Moreover, it is independent of reflectivity and the angular and range weighting functions.

2	15-21	the two sentences beginning with “Because the cited spectral …..”  should be modified to read: “Contrary to accepted usage, the estimates of the second central moment σv2of the Doppler spectrum is not necessarily the sum of the second central moments of individual spectral broadening mechanisms. It has been shown (Fang and Doviak, 2008) the variance associated with shear and antenna motion cannot be separated into a sum of second central moments, and moreover there is an additional term associated with the cross product of turbulence and shear. But if turbulence, hydrometeor oscillation/wobble, and terminal velocities are locally homogeneous (i.e., statistically homogeneous), and estimates are averaged (i.e., spatial and/or temporal), the expected σv2 can be expressed as the sum

,				(5.67)

A rigorous derivation of the spectrum width equation, for non-homogeneous conditions and nearly horizontal beams so terminal velocity is negligible, is given by Eq. (B.13) in Fang and Doviak (2008). Because Doppler shifts associated with terminal velocities of hydrometeors is independent of wind, the second central moment (i.e.,) due to variance in terminal velocities (Section 8.2) of different size hydrometeors has been added to the equation given by Fang and Doviak (2008).

In (5.67), is related to changes in weather signal sample correlation because beam location for each sample changes as the beam azimuthally scans at a rate of  (radian/sec). That is, weather signal samples from different resolution volumes (V6) are not as well correlated as those from the same V6. The term is the shear contribution (i.e., from radial, elevation, and azimuth shear) which depends on  because the azimuth shear contribution increases due to an effectively larger azimuthal beam width (Section 7.8).  is due to changes in orientation or vibration of hydrometeors, and is due to turbulence.”

2	21-23	delete the last sentence of this paragraph

117	1	7	at the end of the sentence, “…to the beam center.”, begin a new paragraph

by modifying the lines following the sentence to read: “If there is no radial velocity shear, and if the antenna pattern is Gaussian…..width , and the antenna rotates at an…”

2	1	change this line to read: “Assume the beam is stationary.  We shall prove that the term σsα2→σs2 is composed of three….”

4-7	modify these lines to read: “where the terms are due to shear of vs, the radial component of steady wind, along the three spherical coordinates at r0.  In this coordinate system (5.70) automatically includes…”

9	change to read: “the so-called beam-broadening term;....”

117	3		replace the text in this paragraph up to and including equations to Eq. (5.75) with:

“Spherical coordinate shears of radial velocity vs of steady wind can be directly measured with the radar and it is natural to expressin terms of these shears. If θ1 << 1 (radian), zenith angle θ0 >> θ1, and angular and radial shears are uniform, vs can be expressed as

(5.71)

where

(5.72)

are angular and radial shears of vs.  Angular shear in units of s-1 is defined as the Doppler (radial velocity) change per differential arc length (e.g., r0dφsinθ0).

Angular shear can be non-zero even if Cartesian shears are zero.  For example, if the Cartesian components of wind are constants u0, v0, w0, the angular and radial shears are

(5.73)

Assume V6 is sufficiently small such that reflectivity and the angular and radial shears are practically uniform across V6 and the weighting function is product separable and symmetric about r0. Then we can substitute Eq. (5.71) into Eq. (5.51) to obtain

.     (5.74)

Because lines of constant converge at the vertical, the second central moment of the two-way azimuthal radiation pattern is a function of zenith angle. That is,, where  is the intrinsic beamwidth for a circularly symmetric beam. The intrinsic beam width is that measured in a spherical coordinate system in which the polar axis is along the beam axis. The intrinsic beamwidth is invariant with respect to the direction of the beam. On the other hand , measured in the spherical coordinate system centered on the radar but in which the polar axis is vertical (i.e., the so called radar coordinate system), increases with decreasing .  is the second central moment of |W(r)|2 .  For circularly symmetric Gaussian radiation patterns having an intrinsic beamwidth ,

(5.75)

118	0		after Eq. (5.76) add: “The above derivation ignored effects of beam scanning during the dwell time MTS.  If the beam scans at an azimuth rate α, it can be shown (5.74) should be written as

(5.77)

where , is the azimuthal beamwidth effectively broadened by antenna rotation during MTS, and θ1e(α) is the effective one-way half-power azimuthal width, a function of αMTs (Fig. 7.25).

124	3     1, 5	change “video” to “voltage” and change “signal” to “voltage”.

125	1	1	replace “average” with “expected”

Eq. (6.5)	append to this equation the footnote: “In chapter 5 ρ is the complex correlation coefficient.  Henceforth it represents the magnitude of this complex function.”

4	5	remove the overbar on P, S, and N

126	0	1	change to read: “power estimate  is reduced……variance of the Pk..”

3	2-4	the second sentence, modified to read, “The Pk values of meteorological interest...meeting this large dynamic range requirement”, should be moved to the end of the paragraph 1

5	change “” to “”.

127	0	1-2	remove the overbar on P in the three places

3	1	remove the overbar on Q

8	delete the citation “(Papoulis, 1965)”

128	1	8	change “unambiguous” to “Nyquist”

2	4-7	rewrite the second and third sentences after Eq. (6.12) as: “The variance of the estimates , each obtained by averaging M un-weighted signal power  samples, is calculated  using the distribution given by Eq. (4.7) to calculate the single sample variance in Eq. (6.9) (in using Eq. (4.7) we set  because noise power is assumed to be zero); this gives. Thus the variance of the M sample average is, from Eq.6.10,  where MI is calculated from Eq. (6.12).”

3	1-2	change to read “To estimate S in presence of receiver noise, we need to subtract.....”

4-9	remove overbars on P, N, and S

129	0	5-6	change last sentence to read: “....then the number of independent samples can be determined using an analysis similar to.....”

130	Table 6.1	add above “Reflectivity factor calculator” the new entry  “Sampling rate”, and in the right column on the same line insert “0.6 MHz”. Under “Reflectivity factor calculator”,  “Range increment” should be “0.25 km” and not “1 or 2 km”. But insert as the final entry under “Reflectivity factor calculator” the entry “Range interval Δr”, and on the same line insert “1 or 2 km” in the right column.

134	1	4	change to: “…..are independent, , obtained directly from (6.21), is

Eq. (6.22b)	change approximate sign to equal sign

136   footnote		change to read:

“To avoid occurrence of negative, only the sum in Eq. (6.28) is used but it is multiplied with ”

137	2	1	delete “()”

142	Eq. (6.42)	place a caret

150	1	4	rewrite line 4 to read: “…shift  (is the path averaged wavenumber increment associated the vertically polarized wave propagating through precipitation along the propagation path) and the two-way total differential phase,”

155	3	3	in Section 6.8.5 line 3, change “Because” to “If”

160	2	6	change “unambiguous velocity ” to “Nyquist velocity”

171	0	3	Ts should be T2

173	0	1	change to read: “…velocity interval vm for this….”

Eq. (7.6b)	place before vm

3	9-10	this should read: “…the desired unambiguous velocity interval.  An unambiguous velocity interval vm =…”

11	change “unambiguous” to “Nyquist”

182	Eq. (7.12)	WiWi+1 should be WiWi+l

197	1	1	“though” should be “through”

2	4	“Fig.3.3” should be “Fig.3.2”

200	Fig.7.28	Along the abscissa, change AZIMUTH (O) to ELEVATION (O), and in the caption delete the parenthetical phrase at the end of the caption. The second sentence should read: “Sidelobes with radome are specified to be below the dashed lines. See Supplements for more information.

201	0	2	“Norma” should be “Norman”

Eq. (7.36)	change “P(r0)” to “E[P(r0)]”, and the upper integration limit to 2

213	1		change this paragraph to read: The Marshall-Palmer (M-P) data extend over a relatively short range of drop sizes (Fig. 8.3a). Earlier measurements (Laws and Parsons, 1943; Fig. 8.3b) that span a much larger range of drop diameters show that the drop size distributions (DSDs) at small drop diameters do not necessarily converge to a constant N0 as suggested by Marshal and Palmer. The large increase in drop density at smaller drop diameters is also seen in the theoretical steady-state distributions derived by Srivastava (1971). But other measurements (citation?) show DSD decreasing at smaller drops. Thus the one free parameter (i.e., ) of an exponential DSD is at best a rough estimate of the true DSD.

222 	Eq. (8.18)	the differential “dD” on the left side of Eq.(8.18) must be moved to the end of this equation.

228	1	2-3	change to read “…(Smith, 19084). Assuming Rayleigh scatter, the radar equation…..of water spheres.”

Eq. (8.24)	this equation should read as:

(8.24)

2	6	change to: “..to estimate the equivalent rainfall rate Rs (mm/hr) from the...”

7	delete “with Zw = Ze”

232	0	10-11	change to: “...a microwave (i.e., λ= 0.84 cm) path, confirmed....”

234	Eq. (8.30)	right bracket “}” should be matched in size to left bracket “{”

240	2	9	here, and at (240, 3, 3) and at (241, 0, 9) change “…coefficients…” to “… “…matrix.elements….”

242	3		insert after the first sentence: All expectations of the matrix elements are per unit volume (i.e.,  in Eqs.(8.46) is ). Thus in (8.46) is not the same as that defined in Eq.(8.45).

243	0	6	delete “the scattering coefficient”

244	2	1	change to read: “…phase shifts of the propagating wave can have..”

248	Eq. (8.57)	parenthesis “)” needs to be placed to the right of the term “(b/a”

249	Eq. (8.58)	cos2 δ should be sin2 δ; replace ko with k; pv and ph should be replaced with pa and pb respectively

Eq.8.59a, b	change the all the subscripts “h” to “b”, and “v” to “a” in these two equations.

2	9	change to read: “pa and pb are proportional to the drop’s susceptibility in generating dipole moments along its axis of symmetry and in the plane perpendicular to it respectively, and e its eccentricity,”

12-17	rewrite as: “...symmetry axis, and  is the apparent canting angle (i.e., the angle between the electric field direction for “vertically” polarized waves, v in Fig.8.15, and the projection of the axis of symmetry onto the plane of polarization). The forward scattering........[Eq. (8.30)] for drops that do not appear canted are given by fh = k2pb, and fv = k2 [(pa-pb)sin2δ +pb] (Oguchi, .....”

3	2	replace “…coefficients…” with “…matrix elements...”

4-5	rewrite as: “Hence from Eq.(8.58) an oblate drop has, for an apparent canting angle = 0, the following cross sections for h and v polarizations:”

268 Fig. 8.29		LDRhv on the ordinate axis should be LDRvh

0	1, 4	change LDRhv to LDRvh at the two places it appears in this paragraph.

269	Fig. 8.30		in the caption, change LDRhv to LDRvh at the two places it appears.

277	0	16	change “23000” to “230,000”

289	2	3	delete the sentence beginning with “In this chapter overbars….”

298	Fig.9.4a, b 	here and elsewhere in the text, remove periods in time abbreviations (i.e., should be: “CST”, not C.S.T.”)

306	2	2	at the end of the sentence on this line, insert: “Radar measurements of wind are biased by the velocity of scatterers (e.g., hydrometeors, insects, etc.) relative to the wind. In this section we consider scatterers are perfect tracers of the wind but later (Section 9.3.3) we introduce corrections for the bias caused by the hydrometeors’ terminal velocity.”

390 	0	1	change to read “along the path ℓ of the aircraft, and Sij(Kℓ) is the Fourier transform of Rij(). In contrast....”

393	1	11	the subscripts on R11(0) should be changed to Rll(0); (i.e., so that it is the same as the subscripts on the second “D “ in line 19).

Eq. (10.33)	place subscript l on C so that it reads Cl.

394	0	1	change to read: “where  is a dimensionless parameter with a value of about 2.”

Eq. (10.37)	change to read:

(10.37)

398	1	12	change to read: “…of the weighting function In, and Φv(K) is the spatial spectrum of point radial velocities.”

17	change to read: “…antenna power pattern under the condition, θe = π/2 – θ0 <<1, and….”

398	Eq. (10.48)	change the subscripts “ll” to “11”.

404	2	1	change to read: “….proportional to the radial component of turbulent  kinetic energy,…”

4	7	place an over bar on the subscript “u” in the next to last equation

409	3	2	change to read: “….must be interchanged withσr, and the second parameter (i.e., ½) in the argument of F must be changed to 2.

5, 6	change to read: “Using the series expansion for F to first order in 1−σθ2r2σr2, the dissipation rate can be approximated by

412	2, 3	2, 1	delete the word “linear” in these two lines.

2	5	change “polynomial surface” to “polynomial model”

7	change “surface” to “model”

419	Fig. 10.18	the “-5/3” dashed line drawn on this figure needs to have a -5/3 slope. Furthermore, remove the negative sign on “s” in the units (i.e., m3/s-2) on the ordinate scale; this should read (m3/s2).

445	1	6	delete “time dependence of the”

451	Eq. (11.98)	change kz to Kz

453	1	10	delete “(s)” from “scatterer(s)”; subscript “c” in ρc, | | should be replaced with subscript “B” to read ρB, | |

12	a missing subscript on should be subscript “B” so the term reads:

Eqs. (11.105 &106)	the symbols should also be subscripts, along with “B”, on the symbol “ρ” to read “” and “”.

454	0	6	change “blob” and “blobs” to “Bragg scatterer” and “Bragg scatterers”

Fig.11.11	caption should be changed to read:  “........, a receiver, and an elemental scattering volume dVc.”

456	Eq. (11.115)	bold “r” in the factor W(r) needs to be unbolded

Fig. 11.12	add a unit vector ao drawn from the origin “O” along the line “r0”.

458	2	4	make a footnote after to read: z′ is the projection of r′ onto the z axis; not to be confused with z′ in Fig.11.12 which is the vertical of the rotated coordinate system used in Section 11.5.4.

459	Eq. (11.125)	delete the subscript “c” in this equation, as well as that attached to in the second line following Eq.(11.125) so that it reads “”.

460	1	4-9	delete the third to fifth sentences in this paragraph and replace with the following:

Condition (11.124) is more restrictive than (11.106); if (11.124) is violated the Fresnel term is required to account for the quadratic phase distribution across the scattering volume, whereas (11.106) imposes phase uniformity across the Bragg scatterer; this latter condition is more easily satisfied the farther the scatterers are in the far field (also see comments at the end of Section 11.5.3).

464	Fig. 11.14		caption: the first citation is incorrect. It should read: “(data are from Röttger et al., 1981)”. Furthermore, delete the last parenthetical expression: “(Reprinted with permission from ….).”

468	2	11	change “(11.109)” to “11.104”

475 			Eq. (11.151) should be:

478	0	7	change to read:

“...the gain g. Then g, now the directional gain (Section 3.1.2), is related...”

493	1		delete the last sentence and make the following changes:

1) change lines 2 and 3 to read: “... Cn2 = 10-18 m-2/3  (Fig.11.17),  the maximum altitude to which wind can be measured is computed from Eq.(11.152) to be about 4.5 km.”

2) change lines 4 and 5  to read: “…that velocity estimates are made with SNR = -19.2 dB (from Eq.11.153 for Ts = 3.13x10-3 s), and that σv  = 0.5m s-1, SD(v) = 1 m s-1, and a system temperature is about 200 K (Section 11.6.3).”

2	1-4	change to read: “Assuming that velocities could be estimated at SNRs as low as -35dB (May and Strauch, 1989), the WSR-88D could provide profiles of winds with an accuracy of about 1 m s-1 within the entire troposphere if  values…”

8; 9	change ‘“14’ to ‘12’; change “able to measure” to “capable of measuring”

533		3	change “Hitchfeld” to “Hitschfeld”

SUPPLEMENTS

The following supplements are provided at the indicated places to clarify and/or extend the text of “Doppler Radar and Weather Observations”, Second Edition-1993; 3rd and 4th printings.

Page	Para.	Line	Remarks:  Paragraph 0 is any paragraph started on a previous page that carries over to the current page.

xv	4	1	change “meteorology” to “meteorological applications”

List of Symbols

Add:

E[x]		Expected value of the random variable ‘x’

x		An M sample estimate of E[x],

x		Spatial average of ‘x’

Kw		The complex dielectric factor of water, p.35.

k0		Electromagnetic wavenumber in vacuum

<p>		A sample average of the property “p” of the members

Amend:

k	Electromagnetic wavenumber () in the atmosphere

3	3		at the end of this paragraph add: R. D. Hill (1990) cited work done by researchers at the Naval Research Laboratory suggesting the first pulse radar detection of aircraft might have been in December 1934.

11	0 	6	modify to read: “….earth’s atmosphere (e.g., the speed v of the wave in the atmosphere is slightly slower than its speed c in vacuum).”

18	1	10-11	change to read: “….a is the earth’s radius (approximately 6375 km), and R…center of the earth to a point on the ray (Fig.2.5).”

19	1	3	amend to read: “…R= a + h (Fig.2.5).”

Eq.(2.24b)	rewrite as

(2.24b)

1	16	change to read: “where rc is the radius of curvature. If n is well….”

Fig.2.6b	Because this figure is also used to derive Equations (2.28) which give the height hs and arc distance s of a scatterer S in terms of radar measurements (), this figure would be more useful if it had an additional ray directed at an elevation angle . S is located along this ray at an arc distance s. Scatterer S is seen by the radar along an apparent straight line (i.e., the ray path) at an elevation angleand slant range sr = r. How r is related to the measured apparent range ra is given in by Eq.(A.7) of Appendix A. The following additional changes to Fig.2.6b are:

(1) change he to h0 to emphasize h0 is the height of the ray at A when = 0o; (2)  place the characters “R” at the radar location and “A” at the end of the ray RA; (3) extend the radial line ae to S; (4) label this extension hs; (4) label the center of the equivalent earth as Oe; and (5) show the angle which is subtended by radials to R and S from Oe.

Also, to Fig. 2.6a, add the character “R” at the radar location. The caption to Fig.2.6 should read:

Fig.2.6: (a) Circular path of a ray launched at an elevation angle from the radar at R on a spherical earth of true radius a. (b) Straight path of the same ray above an equivalent earth of radius ae. Also shown is the straight path of a ray to a scatterer at S at height hs and arc distance s observed by radar at a range r and.

21	0	7	add the parenthetical entry “(Section 2.2.3.2)” at the end of the sentence.

1	1-8	change to read: “Next the radius ae of the equivalent earth will be related to the refractive index gradient. For our purposes, the solution..…to first order in is a good approximation because, for weather radar applications, ranges to precipitation are less than 500 km, and thus  (all angles are in radians unless otherwise noted). Substituting this solution into Eq.(2.25)…. we find that

,

is the height of the ray launched at  above the earth of radius a. To obtain the height h0 of a ray launched at the same = 0o, to slant range sr above an equivalent earth of radius ae (Fig.2.6 b), apply the Pythagorean theorem to triangle OeRA and relate sr to  to obtain the two equations which can be solved giving . Because, and , the approximation

is acceptable. Equating h from Fig.26a to h0, using the relationfrom Eq.(2.24b), and solving for ae, we obtain

(2.27a)

where ke is the equivalent earth radius factor. For the first one or two kilometers of the atmosphere it is customary to set dn/dh = -1/4a (Bean and Dutton, 1966; section 1.5) so in this case the effective radius of the earth is

(2.27b)

13-21	Because Eqs.(2.28b,c) can be derived directly, Eq. (2.28a) is unnecessary.. To demonstrate this derivation, Fig.2.6b needs to be updated as shown in this supplement. Thus delete Eq. (2.28a) and modify the text as:

“…..determine the height and arc distance s to a scatterer at S (Fig.2.6b). In this case all rays launched at are straight and trigonometric identities can be used to relate the radar measured  and slant range sr = r to scatter S to obtain the height hs and arc distance s to the scatterer. Range r is not directly measured, but the range-time delay  of an echo is measured. Range-time is the time delay, after the time a microwave pulse is transmitted, that an echo is received (Section 3.4.2).

The relation for hs is obtained by applying the law of cosines to the angle at R of the triangle OeRS (Fig. 2.6b). Then it can be shown

.		(2.28a)

Thus given the range r and elevation angle  of an echo, the scatterer’s height can be calculated using (2.28a). The scatterer’s arc distance s can be related to r andby applying the law of sines to angles at R and Oe of the triangle OeRS and noting that . Thus

.				(2.28b)

By substituting for hs from (2.28a) we have a solution for s in terms of the radar measured parameters r and. Appendix A gives exact solutions to the arc distance s and height hs of a scatterer in a spherically stratified atmosphere if the refractive index is any known function of height h.

21			delete the last three lines.

22	1	4	change to read: “…<< 1 which appears to limit the elevation angle to small values. But shortly it will be shown that the effective earth radius model can be used to higher elevation angles.”

12-13	change to read: “….given by the Central Radio Propagation Laboratory’s exponential reference model, one that agrees closely with measured data. Although there are several pairs of Ns and H entries, the pair Ns = 313 and H = 6.95 km are the climatological average for the United States (Bean and Dutton, 1966, Section 3.8).

23	0		At the end of this paragraph add: “Although we use the 4/3rds earth radius model (i.e., ke = 1.33) for this comparison, ke = 1.4 is derived from the exponential model. If we use this ke for an equivalent earth radius there is less than 300 m height difference (vs. 500 m for ke = 1.33) between the two models at s = 250 km.  But ke = 1.2 is used by the NWS to determine the height of the WSR-88D’s beam above the earth. In this case the height difference between the exponential model and the equivalent earth radius model is 900 m at 250 km.

31	Fig. 3.1	There are many definitions for a spherical coordinate system. In this book we attempt to consistently use the one given by the International Organization for Standardization (i.e., ISO) and also used by J. Stratton in his book on Electromagnetic Theory (1941, McGraw-Hill Book Co.), as well as by C. A. Balanis in the third edition of his tome “Antenna Theory” (2005, John Wiley & Sons, Inc.).  In this spherical coordinate system, the polar axis is along z (i.e., in the vertical direction), the x axis is Eastward, and the y axis is Northward. The spherical angleis the zenith angle (also called the polar angle), and  (or) is the azimuth measured counter clockwise (when viewed in the negative z direction) from the x axis. However, in we also useas the angular displacement from the beam axis. Moreover, the azimuth (often designated by “Az”) is defined by radar meteorologists as the angle measured clockwise from the y axis—that is, Az = . Finally, meteorologists consistently use the beam’s elevation angle, designated herein as =90-.

32	1	4	strictly power density incident on a scatterer at () is also a function of beam direction (,). For conciseness we do not explicitly show beam direction.

33	1	4	to provide a radiation pattern representative of the WSR-88Ds, change to read: ….levels and, for the WSR-88D, the normalized power density across the aperture is well approximated by

.

34	0	1	delete

2	change the Sherman citation to (Doviak et al., 1998) and replace (3.2a) and the sentence following it with

(3.2a)

where Da = 8.534 m, λ is the wavelength (e.g., λ = 0.1109 m for KOUN a research WSR-88D) and is the polar angle measured from the beam axis, not the zenith angle shown in Fig.3.1.  is the Bessel function of the first kind and of the nth order. This theoretical pattern agrees quite well over most of the angular space except for nine ridges of enhanced sidelobes due to blockage and scatter by the three feed support spars (Doviak, 2017). When the beamwidth is small compared to ……

13	change to read: ….and the angular diameter of the first null circle (the first null circle is a minimum not a zero) is given by. The angular diameter of the first null circle defines the main lobe or beam of the antenna. It has been shown, (3.2b) holds for all WSR-88Ds (Doviak et al., 1998)

2	6	insert; If the shape of the radiation pattern of a beam, not necessarily circularly symmetric, is well approximated by the product of two Gaussian functions, the maximum directivity is

gt'=1σφσθ,

where σφ2and σθ2 , assumed to be much smaller than 1 rad2, are the second central moments of the two-way pattern expressed in normal form. The two-way pattern is the product of the transmitted radiation pattern and the receiving antenna’s field of view pattern. Typically the same antenna is used for both transmitting and receiving, and for circularly symmetric Gaussian shaped pattern, the two-way pattern isf4(θ)=exp−θ22σθ2.  In terms of the one-way 3-dB pattern widthθ1, σθ=θ14ln2(for more discussion see Section 5.3).

35	0	1-2	at the end of this sentence add: Strictly the power gain pattern is the pattern of gain in power density.

1		There are several definitions of cross sections.  For example,  is the differential scattering cross section; that is, it is the cross section per unit solid angle.  Integration of over 4π steradians gives the total scattering cross section  (Section 3.3).

4	6	Kw is used quite often but has not been given a name, it is recommended to call it the “complex dielectric factor of water”. Thus rewrite this line as: “where Kw, the complex dielectric factor of water, = (m2 +…..”

36	0	2	insert at the end of the first sentence: “It can be shown, using formulas presented in Section 8.5.2.4, that Eq. (3.6) has practical validity only if drops have an equivalent spherical diameter De less than 2 mm. Drops having De larger than 2 mm have backscatter cross sections differences larger than about 0.5 dB for horizontally and vertically polarized waves (i.e., σbh>1.1σbv).”

38	1	4	add at the end of this paragraph: “Furthermore, Probert-Jones (1984) demonstrated that internal resonances in electrically large low-loss spheres can generate greatly enhanced scatter in both the forward and backward directions.

2	1	change to “…by water or ice hydrometeors, radars with shorter…..”

2-4	to be consistent, change “drop(s)” and  “particle(s)” to “hydrometeor(s)” everywhere in these paragraphs.

5		change to read: Assume Si  incident on hydrometeors within an elemental volume does not significantly change across this volume. That is, the field in  is assumed to be the same whether hydrometeors are present withinor not. This assumption is referred to as the Born (or single-scattering) approximation, In this case the power density change of a wave…..”

39	0	7	change to “….density at range r2 given is,”

13-14	change to “….coefficient, and is the extinction cross section. , named the hydrometeor size distribution, is the expected number density of hydrometeors per unit diameter having the extinction cross section . Specific attenuation is a function of r both because hydrometeor density can change from location to location and because hydrometeor structure (i.e., liquid or ice water and its shape), can change with r (i.e., can also be function of r). Consequently the specific attenuation k is, in general, a function of r.  The product  gives…”

1	5	change to read as: “…where  is the one-……loss factor due to…

42	Fig. 3.5	caption: Because there is considerable confusion concerning the use of the unit dBZ, and because some writers use dBz for the decibel unit of reflectivity factor Z, we present the following comment:

The logarithm decibel dB is not an SI unit. But dB has been accepted widely as a “unit” (e.g., Reference Data for Radio Engineers, 5th Edition, Howard W. Sams, publisher, division of ITT, p.3-3).  The decibel is also recognized by international bodies such as the International Electrotechnical Commission (IEC). The IEC permits the use of the decibel with field quantities as well as power and this recommendation is followed by many national standards bodies. Moreover, according to SI rules, units should not be modified by the attachment of a qualifier. Nevertheless, appendages to dB have been accepted in the engineering field to refer the dB unit to a reference level of the quantity being measured. For example, dBm is the decibel unit of power and is equal to 10 log10 P/Pr where P is the power, in milliwatts, referenced to Pr = 1 milliwatt (e.g., Reference Data for Radio Engineers).  Similarly, the parameter dBZ has been accepted by the AMS as the symbol for the “unit” decibel of reflectivity factor Z referenced to reflectivity factor Z = 1 mm6m-3 (Glossary of Meteorology, 2nd Edition, 2000, American Meteorological Society).

43	3	6	for clarity of units revise to read: ‘…therefore specific attenuation (m-1)..’

44	3	4	Blake has more recently published (1986, in “Radar range performance analysis”, 2nd ed., ARTECH House, Norwood, MA.) new values of attenuation in gases. For example, at λ = 10 cm, r = 200 km, θe = 0o, the two way loss is about 0.3 dB larger than that given in Fig.3.6.

47		Table 3.1	1) The entry for the 6 dB bandwidth B6 for the short pulse “Filter” is not matched to the short pulse. The matched B6 for the short pulse is 1.04/ (Eq. 3.39) = 0.66 MHz.

2) The reflector diameter is 8.534 m (28 feet) and has a focal length to diameter ratio f /D = 0.375.

48	2	8	for clarification change to read: “…STALO connected to the detector; in this case the detector—only one is needed---is a video detector and its output is the envelope of the rf echoes) when…”

49			In Fig.3.7 and its associated text we introduce the term ‘range time’, but elsewhere we connect ‘range’ and ‘time’ by a hyphen (e.g., Fig.4.1). To be consistent, and because range and time in the context of these figures are different representations of one and the same variable, it is suggested that everywhere it applies, we should use ‘range-time’ (ditto for ‘sample-time’).

56		Eq. (3.34)	if the beam is passing through clouds and storms, Eq. (3.34) should be replaced by

whereand Tc are the cloud’s attenuation and temperature.

58	1	4	change to read: “….effects is reached (Section 4.5).”

57	Fig. 3.11	for completeness, the ordinate should be labeled “Sky noise temperature Ts (K)”

60	1	2	change to: “…noise power N. This peak signal-to-noise ratio, S(0)/N, readily obtained…..

(3.38)		change SNR to S(0)/N in this equation and on line 5.

7	change to: “…that the signal-to-noise ratio SNR is maximized…”

64-121			because Chapters 4 and 5 focus attention mainly on weather signals, changing power P to signal power S throughout these chapters should improve clarity. This suggestion is made because P is generally used to define signal S plus noise N power.

Fig. 4.2 caption	 change last line to read: “…at (Section 4.4.2). This figure assumes the transmitted pulse is initiated at exactly the same time as the timing pulse; more commonly there is a transmitted pulse delay  between the timing and transmitted pulses (see revised figures 4.5, and 4.6).

61	Eq. (3.40b)	elsewhere in the book (e.g., pp.172, 173), we define as the Nyquist velocity. It would be less confusing if is replaced with throughout the book; this change keeps one from callingthe unambiguous velocity which is confusing because all velocities in the interval are unambiguously measured.

68  	Eq.(4.2)	express this equation as:

where proportionality constant.

68	3	8	add: “In most of this book“ the expected value E[x] is assumed to be obtained by averaging over an ensemble of scatterer configurations ξhaving the same statistical properties (e.g., reflectivity factor, velocity field, etc.). But the expectation operator E[x] is used for averages of ensembles of other variables (e.g., , scatterer cross sections in  Section 4.4). To avoid subsequent possible confusion, we could have appended the subscript ξ to haveEξ[x], but the subscriptξwill be used only where there is a need to avoid confusion.”

71	2, 3		an explanation for the √2 factors that the errata stated needs to be inserted into Eqs. (4.4) and (4.6) might be helpful.  Because a lossless receiver is assumed, the sum of powers in the I and Q channels must equal the power at the input to the receiver (i.e., the synchronous detectors in Fig. 3.1).  Because we have assumed the amplitude of the echo voltage at the receiver’s input is A (e.g., Eq. (2.2b)), the amplitude of the signal in the I and Q channels must be A/√2.  Furthermore, we can determine from Eq. (4.5) that the rms values of I and Q voltages equals σ (i.e., Irms = Qrms = σ). Thus the average power in each of the channels is σ2, and the sum of the average powers in these two channels is 2 σ2 which equals the expected power E[P] at the input to the receivers.  The constants of proportionality (i.e., impedances) that relate voltage to power are assumed the same at all points in the receiver (e.g., at inputs to the I and Q channels).

72	2	2	here and in Eq. (4.8), we need to distinguish the expectation operator “E[x]” used in Sections 4.2 and 4.3, which implicitly applies to an ensemble of scatterer configurations, from the expectation over the scatterer’s cross section given by Eq.(4.8). To emphasize this difference, we rewrite Eq.(4.8) and revise the text as:

“….and the expected backscattering cross section. The expected power contribution  from each scatterer to  is,

(4.8)

where is the ith scatterer’s expected echo power which can be directly….…using Eq.(3.24). The subscript ‘’ is appended to ‘E’ to differentiate this expectation from Eξ[x]which is an expectation made over the configurationsof scatterer locations (Supplement 68, 3, 8).

Sample time averages provide estimates of both expectations, because hydrometeors change their cross sections (due to wobble and oscillations) simultaneously and they change their relative locations. Thus, henceforth the expectation ‘E’ without subscript denotes the combined average over all variables that cause the echo voltage to fluctuate. An average over the ensemble of backscattering cross sections is needed because a hydrometeor undergoes continuous…….as it falls through the air. Thus its backscattering fluctuates about an expected value. Therefore the expected value (i.e., …is needed to determine”

Eq.(4.9a)	To explicitly differentiate between expectation ‘E’ over all random variables that cause weather signal fluctuations, and the expectation of the hydrometeors’ cross sections, rewrite this equation as:

(4.9a)

73	2	3	To account for drops not being spherical change to:

“…..per unit volume. In this derivation it has been assumed that drops are spherical, but in fact they are oblate (Fig.8.1). Thus the backscattering cross section depends on the polarization of the wave. If waves are horizontally or vertically polarized, the backscattering cross section is noted as either or  and reflectivity as either or (more detailed discussion is given in the supplements to pp. 240-242).

74	1	4	change to read: “….to its range extent, and if the polar axis of the spherical coordinate system is aligned with the beam axis (i.e., θ0 = 0, Fig. 3.1), we can approximate Eq. (4.11) by…”

8	change to read: “…by a Gaussian shape, where , we can show…”

74	Eq. (4.14)	to be have left and right sides of this equation consistent, write reflectivity and loss factor as and

10	change to read: “where  is the …. of the one-way power pattern.”

76	0	1	change to read: “…is, assuming a shell of thickness , scatterers in  that shell return an echo voltage”

4	insert the sentence: Eq. (4.17) assumes and scatterers do not introduce any phase shift upon backscatter (i.e., scatterers have negligible phase shift).

Eq. (4.18a)	the text would read better if Eq. (4.18a) and the sentence containing this equation were moved to follow the first sentence in paragraph 1.

1	6	to have consistency between the power response  and its 6 dB bandwidth B6 (a parameter commonly used to define a “matched condition”) change this line to read: “….is rectangular and  has a Gaussian shape with a 6 dB bandwidth, B6 equal to , the reciprocal of the pulse width, ….”

1	12-15	because the comment about causality is not consistent with Fig. 4.5, and to include the time delay in arrival of the transmitter pulse at the antenna aperture relative to the start of the timing pulse (Fig. 4.2) and the time delay of the received echo propagating from the antenna aperture to the receiver output, Fig. 4.5 should be modified as follows:

Insert the symbol between the end of the functionand the tick mark at. To designate the start of the timing pulse, place another vertical line to the left of the existing one (this additional line designates the start of the transmitted pulse) and insert the symbolbetween it and the existing line. Third, extend the horizontal line markingto the new vertical line. Finally insert the symbol between the point at which rises on its right side and a vertical line through the peak of .

Furthermore, lines 15, 16 should read as: “…are obtained (Fig. 4.5), accounting for: 1) the transmission line delayof the echo’s arrival at the antenna and its appearance at the output of the receiver, and 2) the time difference between the start of the timing pulse and the  arrival of the transmitted pulse at the aperture (Fig. 4.5), and substituting =….”

After Eq, (4.18b) write: In Fig.4.5,is the time required for the received echo to propagate from the antenna aperture to the receiver output, is the time, after arrival of the scatterer’s echo at the receiver’s output, for the echo to reach a maximum amplitude, and is the time at which the transmitted pulse arrives at the antenna aperture after the start of the timing pulse (note that  can be positive or negative).

77	0	5	insert after Eq.(4.19): In writing Eq. (4.19), time delaysand are assumed to be zero; otherwise the upper limit should read as .

78	0 	2	for consistency, here and everywhere in this section including Fig. 4.6, change to.

Fig. 4.6 	insert the symbol to designate the time between the arrival of the echo at the receiver’s output to the time the echo is a  maximum.

0	4-5	change to read: “Thus only if…and if transmitter delay  is zero, would…”

1	3	change to read: “….delay is the sum of the ….”

10	change to read: “estimate the range to scatterers that are contributing most to the echo sample (this assumes scatterers are all of the same cross section). That is, a voltage sample….”

79	0	8	add at the end of this paragraph: Range r0 is the range to the center of the radar’s resolution volume defined in Section 4.4.4.

80	2	2-3	change to read: “….The 6 dB angular width  of …function) is taken to be the angular width of V6 because it equalsthe 3 dB width of the one-way power pattern, a commonly accepted definition of angular resolution.

81	0	11	change to read: “….width , and…”

82   	1		Eqs. (4.31) and (4.32) without the subscript “w” apply to all spherical particles, not only water. Thus to emphasize this point, and to make these two equations consistent, modify line 3 to read:

“If scatterers are spherical…”.

Then delete the subscript ‘w’ in (4.31) and (4.32) and rewrite (4.31) and the line that follows as:

,					(4.31)

where Km is the dielectric factor of the medium filling the sphere and

(4.32)

is the reflectivity factor of spheres. For water spheres the subsctip ‘w’ is appended to K and Z. Whenever the Rayleigh….

83	Sections 4.2	Change heading to “The Per-Pulse Signal-to-……” and the first line to “The per-pulse SNR for distributed……”

2		For further insight and interpretation into the discussion in this paragraph add: “This result can be looked upon as being similar to a generalized uncertainty principle. Here it is applied to the realization that we cannot simultaneously concentrate the signal f(t) in the time domain and its spectrum S(f) in the frequency domain. A measure of concentration are the second central moments and , and these two measures satisfy the condition

.

Equality is achieved only for the Gaussian pulse (Harris, 1978).”

89	3	6-8	To extend the discussion to include M being odd rewrite these lines as follows: delete ‘(M/2)’ on line 6, and rewrite lines 7 and 8 as:

“..done in order to have an equal number of positive and negative Doppler lines to the right and left of the zero (k = 0) line. If M is even, the power is split so there…”

92	3	1	To alert the reader of the transition to weather signals, insert the following at the beginning of Section 5.1.2 and modify the first sentence as shown:

Voltage V in the preceding sections is not necessarily a random variable as is the weather signal. In this and the following sections, the focus will be on random voltages. The Fourier transform…..and correlation of sample sequences of the random voltage generated by weather echoes.  The output …….

94	2	1	statistical properties of a voltage sample sequence are, for example, its expected mean square value (i.e., proportional to expected power), its autocorrelation function, etc.

95	0		this paragraph should be clearer, and better connected to the properties of the scatterers, by revising it to read from the 4th to the 10th lines as:

“…the same environmental conditions (e.g., the same wind field). Although the environmental conditions might not change, the configurations of the scatterers during another observational period can be altogether different, and the changing scatterer configuration will produce another sample sequence both sharing the same statistical properties (e.g., the reflectivity). The statistical properties of weather signals can be estimated reasonably well from one member of the ensemble of all possible sample sequences if the single sample sequence is of sufficiently long time duration (for weather radars, this duration is typically >0.05 s). If the statistical properties of the ensemble can be deduced from samples from one member of the ensemble of sequences, the samples V(m) are said to be ergodic. Only then can the properties of an ensemble be inferred from processing V(m)s. Sample sequences produced by scatterers with statistically stationary properties are members of an ergodic ensemble. We shall assume…..”

Eq. (5.17)	note that this equation is a biased estimate of the autocorrelationR(l). To obtain an unbiased estimator, replace the M -1 multiplier with (M-1-|l|)-1.

106-112		because Sections 5.2 and 5.2.1 stipulate that the velocity field is steady (i.e., time independent, and to be consistent with later notation (i.e., errata on p. 113, line 1), we could have, in these two sections, appended the subscript ‘s’ to ‘v’ to emphasize that ‘v’ is a steady wind. Moreover, should not be confused with the one-way half power beam width. That is, variation is limited to the volume over which radial velocity is steady.

110	Eq. (5.50)	is sometimes defined as the variance of the mean normalized Doppler spectrum. In that case the Doppler spectrum is interpreted as showing the probability density of velocities. This is at best a questionable interpretation, because Doppler velocities are a composite of real and apparent velocities (Fang and Doviak, JAOT, Dec. 2008, pp. 2245-2258).

112	2	10-12	change to read: “…turbulence, etc.) act independently as we now demonstrate.”

113 footnote 5		Add to the footnote: is only the weight impressed on the ith scatterer if the antenna is not scanning. Because the antenna pattern on transmission is in a different direction on reception, for azimuthally scanning antennas, .

114	0	8	change to read: “…where is the expected backscattering cross section of the kth hydrometeor (expectations computed over the ensemble of cross sections for the kth scatterer), it follows …

116			at the end of Section 5.2, add the following paragraph:

In this section we assumed scatterers follow exactly the air motion. But usually scatterers are hydrometeors that fall in air, have different fall speeds because of their different sizes, and change orientation, and vibrate (if they are liquid). These hydrometeor characteristics broaden the Doppler spectrum associated with the velocity field increasingobtained from Eq. (5.51).

2	6-8	Change to read: “…of scatterers, shear of vs can cause ….as can differences in the terminal velocities of various….”

117	1	1	Change to “…components of terminal velocities of the assorted-size…”

118	0		after Eq. (5.75): It should be noted that as θ0 → 0, the angular shears in Eq. (5.74) should be replaced by kθ along the two principal axes of the beam pattern. For example, if the beam is circular symmetric and θ0 = 0, .

after Eq. (5.76): It should be noted that if the receiver bandwidth B6 is much larger than the reciprocal of the pulse width τ,.

124-131		to be consistent with notation used in Chapter 5 and that used after page 131, we should change ‘k’ to ‘m’ and ‘m’ to ‘l’ wherever it appears in these seven pages.

125	2	1	change to read: “It was explained in Section 5.2.2 that weather….”

5	change to read: “….assume a Gaussian signal power spectrum plus white noise,

Eq. (6.3)	because is the signal plus noise power spectrum, and because P generally defines signal S plus N power (e.g., 3rd line from bottom of this page), it is suggested  be replaced with.

Eq. (6.5)	add subscript ‘s’ to ‘’ to distinguish the correlation coefficient of the weather signal from the correlation coefficient of weather signal plus noise to be introduced by Eq. (6.13d) of this supplement.

128	0	2	for clarity, replace, here and elsewhere, with .

2	4	to place more specificity on the values of M and for which the equation is valid, it can be shown that the condition  applies, or in terms of,

,

whereis the Nyquist velocity (i.e., ). Thus the larger is M, the smaller is the allowed spectrum width to use the simplified expression. Nevertheless, this range of spectrum widths covers most conditions observed with 10 cm wavelength weather radars.

Eq.(6.13): 		this equation is valid when signal power is much stronger than noise power. Furthermore, in the fourth line of the paragraph leading to this equation, the SD of  is only proportional to  if the number of independent samples of P and N are equal; in general this is not the case. For example, noise power in the WSR-88D is estimated independently of the signal power, and its estimate uses many more samples than that used to estimate signal plus noise power. Therefore, the following text, replacing paragraph 3 on p.128, gives the standard deviation of Z(dBZ) estimates as a function of Signal-to-Noise ratio assuming noise is estimated without significant variance.

To estimate reflectivity factor Z in presence of receiver noise, receiver noise power N needs to be subtracted from the signal plus noise power estimate (i.e., ). Thus the Z estimate is  whereis a uniformly weighted M sample average estimate of the power P at the output of the square law receiver (as in the WSR-88D), and α is a constant calculated from the radar equation. N is usually measured during calibration but can also be estimated in parallel with weather data collection (Ivić et al. 2013). In both cases, many more samples are used to obtain its estimate. Therefore its variance is negligibly small compared to that of , and the noise power N is nearly its expected value.  Z is usually expressed in decibel units; that is, where units are mm6 m-3.  The error in decibel units is now derived.

Let , the M sample estimate of signal power, be expressed as  where is the displacement offrom S. Thus

Because  where N is a constant, =(i.e., Eq.6.10). If the equivalent number of independent samples MI is sufficiently large so that  is small compared to 1, the second log10 term in Eq. (6.13a) can be expanded in a Taylor series. Retaining the dominant term of the series, the estimated reflectivity is well approximated by

(6.13a)

(6.13b)

The first term and the constant 4.34 are not random; thus the standard error in the estimate  is simply = 4.34. Because the signal power estimate  is derived by subtracting the noise power N , a fixed number, from the measured power estimate , the standard deviation of  is   where MI is the number of independent signal plus noise power samples,

(dBZ).				(6.13c)

The MI contained in the M sample set, can be calculated from (6.12) in whichis replaced by the magnitude of the correlation coefficient of the signal plus noise samples, which is calculated in the following paragraph.

The correlation of the input signal plus noise (Eq. 6.4) is normalized by S + N to obtain the correlation coefficient of the input signal plus noise power estimates. Because is the square of, the correlation coefficient of the input power samples is,

.		(6.13d)

Then can be substituted into (6.12) in place of, and the sum numerically evaluated to obtain MI. If this result is substituted into (6.13c) we obtain a solution for the standard error (in dBZ units) of the reflectivity factor as a function of the number of samples M, the normalized width,of the weather signal’s Doppler spectrum, and the signal to noise ratio S/N.

Alternative to a numerical solution, there are conditions whereby we can obtain a useful analytical solution. It can be shown the condition  still applies to replace the sum in Eq. (6.12) by an integral even if; furthermore, becomes a sufficient condition to ignore the term in that equation. If the dwell time MTs is much longer than the correlation time of the weather signal samples,  and thus the limits on the integral can be extended to infinity. Evaluation of this integral under these conditions yields

(6.13e)

The formula for calculating the standard error in estimating Z(dBZ) as a function of S/N is obtained by substituting (6.13e) into (6.13c) yielding

,		(6.13f)

a result agreeing with that obtained from simulation (Melnikov, 2011). Although  increases without bound as S/N decreases, it can be shown SD of  estimates, expressed in linear units (e.g., mm6 m-3), decreases as S/N decreases and reaches a minimum value at S = 0 (i.e.,  if S = 0; in this case= 0)! This apparent dichotomy is explained by the fact that the slope of the logarithmic function increases without bound as its argument decreases. That is, a constant width of a distribution on the linear scale increases on the logarithmic scale as the mean decreases.

133 Eq.(6.20a)		A slightly more rigorous condition to replace Eq.(6.20a), one based upon the correlation function for the linear receiver (condition 6.20a was derived based on power estimates using a square law receiver) and estimation of the first and second moments, is: .

136	4		At the end of this paragraph add: Furthermore, Eq.(6.29) estimates well the spectrum width for any shape spectrum provided Ts is much smaller than the signal’s correlation time. This is so because the second central moment of the Doppler spectrum is the derivative of the autocorrelation function at zero lag, and because under the stipulated condition the derivative is well approximated by the finite difference formula of Eq. (6.29).

Footnote		Because SNR is used when the signal to noise ratio  S/N  is expressed in dB, change  to .

Figs. 6.6 and 6.7	Because S/N is a ratio of numbers, and because when ratios are expressed in dB, the nomenclature SNR is used, S/N in these figures should be replaced with SNR.

150			To distinguish kh and kv, the incremental wavenumbers, from the free space wavenumbers kh and kv, the free space wavenumbers of the H and V polarized waves which are vastly different in magnitude, change kh and kv, to .

1	8	to remove ambiguity, insert after the end of the sentence: Here is the total differential phase shift but later (i.e., Section 8.5.2.3 and thereafter) is the differential phase shift associated only with propagation. But, for Rayleigh scatter, so the ambiguity typically is not significant.

195	2	1-3	change to read: “Three parameters of interest…… integration, (2) the effective beamwidth, and (3) the effective pattern shape.”

195-196		to be consistent with notation used elsewhere, and because on these pages we exclusively use the term “effective” instead of “apparent” when we describe patterns broadened by azimuthal scanning, use the subscript “e” instead of “a” on fa(φ−φ0) and a.

196	1	1&7	replace a  with e1.

1		at the end of this paragraph add the sentence: “If αMTs θ1, the effective normalized pattern retains a Gaussian shape, but if αMTs >> θ1, the pattern shape is roughly trapezoidal having amplitude (αMTs)-1, and a one-way half-power width of about αMTs.”

2	3&5	replace a  here and in Fig. (7.25) with e1.

Fig. 7.28 		Some general comments: Specified not-to-exceed sidelobe levels given by the dashed lines in Fig.7.28 applies to the WSR-88D radiation pattern with radome. But patterns without radome are specified as follows: from -26 dB at 2o to -38 dB at 10o, and then the constant level should be at -42 dB. The dashed lines in this figure principally limit the ridge of enhanced sidelobes generated by the feed support spars and should not be construed to apply to the entire angular space.

The pattern plotted is one taken without radome and is one along the vertical plane which does not show increase in sidelobe levels due to spar blockage. But sidelobe levels on the left side (i.e., below the beam) are higher due to spar scatter, and sidelobe levels above the beam agree well to 20o with the theoretically calculated levels. Thus the measured sidelobe levels are considerably below the specified not-to-exceed levels.

203	1	4	Rewrite as “equal to or larger than the reciprocal….”

204	0	7	Change to read “...wires, etc. The clutter spectrum width ......”

210	2	1	because raindrops are not spherical and thus raindrop diameter is ambiguous, change this line to read:  “Raindrops of diameter De <0.35 mm (De is the diameter of an equivalent-volume spherical raindrop) are essentially spherical, and drops up …” Thus change D to De where ever diameters of raindrops are being discussed in chapter 8.

Fig. 8.1		It is worthwhile to include in Fig.8.1, the ZDR values for each drop computed using the procedure given in Section 8.5.2.4 assuming drops can be represented by oblate spheroids. Thus drops of De = 2.70 to 8.00 mm computes to have the following ZDR values: 1.3; 2.0; 3.6; 4.0; 5.5; and 6.3 dB.

212	0		add at the end of this paragraph: More recently Clothiaux et al. (JTECH, 1995) present cloud reflectivities much weaker than those reported by Gossard and Strauch (1983). Clothiaux et al. calculated cloud reflectivity factors using published in situ measurements of mean droplet diameter and liquid water content, and assumed a gamma drop diameter distribution with a shape factor of 18 (i.e., the distribution is nearly normal). Clothiaux et al., found that continental cumulus has Z values ranging from -28 to -15 dBZ (3 cases); -35 to -38 dBZ for continental stratocumulus (2 cases); -35 to -15 dBZ for marine stratocumulus (22 cases); and the weakest reflectivity -52 to -30 dBZ is associated with altocumulus clouds (5 cases).

234	2		remarks:

The fitted exponent b = 4.24 for the power law fit to was obtained using the Gans (1912) approximation for the forward scattering coefficients fh,  fv of oblate drops. Although the calculations were not explicitly shown, the Gans approximation (often called the Rayleigh-Gans theory) is explicitly shown for the computation of the backscattering matrix coefficients presented in Section 6.5.2.4. The Gans approximation is crude compared to more precise computation made using the T matrix.  The T matrix computations indicate the best exponent is b = 4.8 (at the frequency of 2.705 GHz). Thus, using about 47,000 DSD measured with an 2D video disdrometer in Oklahoma and the more accurate exponent b, the following better relation between rain rate and KDP is

R = 44 (2.705 KDP/f)0.822

where f is the radar frequency in GHz.  Similarly a more up-to-date relation for the liquid water content is

M = 2.36 (2.705 KDP/f)0.71 .

These relations are valid up to about 6 GHz. Direct extension to higher frequency (10 GHz) would produce an underestimate by about 10 %.

240	1	3	for clarity change to read: “…we consider linear orthogonal polarization in which either h or v polarized waves are transmitted, but the discussion is valid for any other orthogonal polarization basis. But the typically minimal depolarization of h and v polarized waves propagating in precipitation is the principal reason why the linear h, v polarization basis is favored over circular polarization (and as well over other linear polarization bases) for rain rate measurements made with the WSR-88D (Doviak et al., 1998, 2000).”

240	2		The derivation of Eqs. (8.42) and (8.43) implicitly assumes the antenna is ideal so that the h antenna port transmits only horizontally polarized electric fields (i.e., only  in the spherical coordinate system with polar axis along the vertical z coordinate); likewise the v port only transmits . However, to show the effects of antenna cross-polar coupling, replace this paragraph with the following:

“An ideal polarimetric weather-radar antenna has two ports whereby one transmits and receives only h polarized fields whereas the other port functions similarly for v polarized fields. However for practical antennas, including those commonly used for weather observations, if the h or v port is energized, v or h cross-polar radiation is also transmitted. Likewise, backscattered h or v polarized radiation can also appear in the v or h receiver channels. Cross-polar radiation from the antenna can strongly limit the accuracy of polarization measurements (Zrnić, et al. 2010). To show the complications introduced by cross-polar coupling in the antenna, and to provide the basics for computing the effects of cross-polar coupling, assume the h and v ports are alternately energized every other PRT, but are simultaneously receiving both polarizations (i.e., the Alternate Transmit Simultaneous Reception—the ATSR mode of data collection). By substituting equations (2.3) and (3.4) into (8.39), it can be shown the electric field backscattered by the nth hydrometeor at rn is

,		(8.42)

where subscripts j, iT and  = v or h. To shorten notation we have replaced the angular weighting function with. The first index on g and f designates the polarization of the transmitted electric field, whereas the second index j designates the antenna’s transmitting port. The first index ondesignates the backscattered radiation having polarization ‘’ whereas the 2nd index designates the polarization of the transmitted wave incident on the scatterer.is the rms electric field having polarization  backscattered by the nth hydrometeor; all other terms have been defined previously. Precipitation along the path between the hydrometeor and the radar is assumed to be absent or to cause negligible attenuation and differential phase shifts of the propagating waves.

The rms electric field, incident on the hydrometeor is Eq. (8.42) multiplied by . Thus the convention regarding the scattering elements in Eq. (8.42) agrees…… (1975) in which  =. is the expected backscatter cross section that returns h polarized fields incident on the scatterer as h reflected fields. Similar interpretations can be deduced for each of the scattering matrix elements. For convenience we now use angle brackets <x> to denote expected value E[x] used in earlier chapters. But here, averaging to obtain is made over the distribution of the scatterers properties and this will be discussed more fully later when we present (8.45).

To show how to interpret (8.42), consider  and iT = h. In this case the h port on the antenna is energized and the copolar backscattered h field is due to the antenna transmitting the copolar H field echoed by the scatterer’s shh matrix element. But there is another contribution to the h backscattered field. That is, when the h antenna port is energized, a cross-polar v field is also transmitted (i.e., via the term fvh), and this cross-polar field is converted by the scatterer’s shv matrix element to generate an additional h copolar backscattered field. This copolar backscattered field, generated by the combination of cross-polar terms, is typically much weaker than one generated by the copolar terms because fvh (as well as fhv not necessarily equal to fvh) and shv = svh are much smaller than their corresponding copolar counterparts (i.e., fhh, fvv, and shh, svv). Although might not equal it can be shown reciprocity for antennas (Section 3.4) is not violated. Because this backscattered h field component is a second order product of cross-polar coupling terms (i.e., shvfvh), it typically can be ignored and only in this case the h receive channel can safely be said to receive the copolar field.

But if the h antenna port is energized, first order (i.e., only a single cross-polar term is present) v backscattered fields (designated as the cross-polar field) are also generated (i.e., = v, j = h, and iT = h or v). This v cross-polar backscattered field is composed of two components: one generated by the product svvfvh and the other by svhfhh. It is the second cross-polar component that is related to the cross-polar properties of the scatterer, but this cross-polar field can be severely corrupted by cross-polar coupling in the antenna.

In conclusion, if the h port is energized there are four components associated with the backscattered h and v fields; two with the copolar h field (one is 0th order in cross polar terms, and the other is of 2nd order in cross-polar terms) and two with the cross-polar v field (both are 1st order in cross-polar terms).”

3		change to read: “The elemental voltage associated with the echo from the nth scatterer and appearing in the h and v channels of the receiver is designated as , where the first index is the polarization of the backscattered field incident on the antenna, and the second index iR = h or v designates which port is receiving echoes (the second index designates the antenna port for both transmitting and receiving). Using equations (2.3), (3.20) and (3.21) it can be shown, other than for a constant receiver impedance factor,

,		(8.43a)

where is the range weighting function (i.e., ; p. 75). If  (i.e., the h port is transmitting and receiving) there are four components that comprise the weather signal in the copolar h receive channel. Only the = h  = h received signal component is 0th order in cross-coupling terms; the three others are second order and usually can be ignored. On the other hand if j = h, and  = v (i.e., the h port is transmitting, but the v port is receiving cross-polar signals) there are again four components that comprise the cross-polar signals. Three of these are 1st order in cross-coupling terms one of which is the desired component related to shv. The fourth component is 3rd order in cross-coupling terms. Thus unless the antenna’s cross-coupling terms fhv and fvh are relatively small compared to shv (i.e., fhv/fhh<<shv/shh) measurements of shv can be significantly compromised.

If both h and v signals are transmitting simultaneously and on reception both channels are receiving (i.e., the so called STSR mode---Simultaneous Transmit and Simultaneous Receive) as done with the WSR-88D, each receive channel will have 8 components of weather signals; one of 0th order (i.e., the copolar signal), three of 1st order (i.e, fhv or shv only appear singularly), three of 2nd order, and one of 3rd order. Because 1st order terms are mixed with the copolar signals, fhv along the antenna’s copolar beam axis needs to be extremely small (i.e., less than 45 dB below the copolar peak) otherwise there can be unacceptable biases in polarimetric measurements (Zrnić, et al, 2010).

To simplify the remainder of this section, hence forth assume  for  and (i.e., no cross-polar radiation is emitted or received by the antenna) and (i.e., the gain and patterns are identical for horizontal and vertically polarized waves).”

241	Eq. (8.43b)	add a subscript j to F(rn), and after this equation insert:

“where. Now the first index on denotes the received channel (h or v) and the second index denotes the transmitted polarization (h or v). is a time varying voltage because scatterers reshuffle their locations due to turbulence and scatterers can oscillate (liquid water drops) and/or wobble thus changing their appearance presented to the incident wave. Samples ofare obtained at each range location every PRT, although it takes two PRT intervals to collect all the polarimetric combinations of . The expected value of Vij is zero because….…”

0	5-9	change to: “….use various second-order estimates  of moments, to characterize the polarized signals (the subscripted angular bracket denotes an M sample average) and relate these to the properties of hydrometeors. If M is infinitely large the M sample average becomes the expected value. Detailed derivation of the relationship …..…….. from Eq.(8.43b) the expected value of the general term is”

(Eq. 8.44a)	at the beginning of this three part equation modify the beginning of the first line of this equation to:

After this first of the three equations insert:

“where is a 4 by 4 covariance matrix, and has been assumed (i.e., the H and V weighting functions are equal).

Similar to the discussion presented by the supplement to p.72 , the expected value is obtained by averaging both over the configurationof scatterers and the distribution of their properties (e.g., size and canting angle) to obtain . However, M sample averages provide good estimates of the expected value for both configuration and property change if the number of independent samples MI (Section 6.3.1.2) is sufficiently large. Moreover, because the jth polarized field is assumed to be transmitted alternately with the lth polarization, and although echoes are received simultaneously in two receivers, not all signal components are received simultaneously to obtain all the elements of. Moreover, because copolar h and copolar v signals are spaced a PRT apart, adjustments need to be made to the polarimetric variables (e.g., Section 6.8.5) to compensate for the loss of signal correlation and phase shifts due to the relative motion of scatterers during the PRT.

Because the phases are uniformly distributed across 2π, the off-diagonal terms of the double sum vanish when the ensemble average is taken over which leads to

Insert here the last two equations of (8.44a) and continue the dialogue

To obtain the last equality the single summation over n is replaced with…..and the resolution volume weighting function at r. The factor is called the backscattering coefficient (m-1).”

241	0		At the end of this paragraph add the paragraph:

In this integral form are elements of the backscattering covariance matrix for a hydrometeor that represents the ensemble of hydrometeors in . Hydrometeors in dV have various diameters, shapes, etc. The ensemble average  is over the properties (e.g., size, canting, etc.) of the hydrometeors that cause changes in sij. The expectation operation is further discussed and interpreted in the discussion of Eq. (8.45b) presented below. It might seem strange to assign the same to drops of all sizes and shapes of scatterers in dV, but it must be realized the size and shape of each scatterer is not known. The best we can do is to estimate the expected size and shape of the representative scatterer for each dV”

- After (8.44b) insert:

Precipitation filled media are anisotropic if scatterers are not spherical. Anisotropic media are those whose propagation characteristics are functions of the direction of propagation (i.e., the direction of the wave normal k and/or the direction of the wave’s electric field vector (i.e., the state of polarization—Budden, Radio Waves in the Ionosphere, 1961, p.47, and P. Beckmann, The Depolarization of Electromagnetic Waves, 1968, p.14). Beckmann points to the differences in propagation as related to different polarizations. Precipitation filled media are anisotropic for both conditions.

There are other ways……

Eq. (8.45)	to avoid confusion with the N associated with the Drop Size Distribution (DSD), and because the scatterer properties can depend on location of dV, replace N(X) with p(X, r). Also label this equation as (8.45a).

242	0		change N(X) to p(X, r) and add, at the end of this paragraph, the following:

As shown in Section 8.5.2.4, the scattering matrix elements for water drops are functions of drop shape, canting angle, etc. But shape is related to De, and if the canting angle of each drop is zero, p(X, r) is only a function of De, the equivalent-volume diameter of a spherical drop, and r. Under these conditions we have the simplified equation

.		(8.45b)

242	Eq. (8.46)	to have consistent meaning for the parameter  given by (8.45a), the multiplicative factorneeds to be inserted into the first two entries (e.g.,  should be). However, for water drops having zero canting, the only property having a distribution is De. Then the number density in (8.45b) cancels the number density in (8.44a) and in this case, and only in this case, the expectation  can be made over the drop size distribution and there is no need to insert the multiplicative drop density terminto (8.46a,b)! But in this case  should be replaced with to emphasize the ensemble average is over the drop size distribution N(De) , and the units of are m-1.

Item 5		change to read: “Copolar correlation coefficient….”

243	0	8	to connect with discussions in Chapter 6, change to read: “…of the scatterer, and where (Section 6.8.2.2). If, the phase of the scattered V field (referenced to the scatterer’s location) leads the incident V field (similar deductions can be made for all the differential phases .

Eq. (8.48)	change subscripts on  and here wherever they appear to and.

2 		change to read: “Analogous to….coefficients (i.e., the copolar/cross-polar correlation coefficients) can be defined as

(8.48a)

(8.48b)

The symbols andare chosen to ….co-polar signal (indicated by the second subscript) and cross-polar signal (indicated by ‘x’), and distinguish these…..between the copolar signals.”

244	0	3	delete “off-“ in the phrase “off-diagonal terms”

0	6	for clarification change to read as:  “….phase shift  upon backscatter and the round trip differential phase shift due to oblate scatterers along the propagation path, it is…”

Eq.(8.49a)	because we state;  “…to write the correlation coefficient in which the two  are separated, it would be clearer if Eqs. (8.49a,b, and c) were expressed as

(8.49a)

(8.49b)

(8.49c)

where and  are phase shifts upon backscatter. In deriving these equations it was assumed propagating waves only have additional mean phase shift due to precipitation along the path to and from the resolution volume V6 (i.e., there are no added random fluctuations in amplitude and phase due to hydrometeors along the path). In short, although the hydrometeors along the path have random motion, oscillate, and wobble, they do not significantly affect the wave incident or reflected from the resolution volume. This conclusion is rooted in our implicit assumption that multiple scatter can be ignored (i.e., the Born approximation; Born and Wolf, 1964 p. 953). Thus, all random amplitude and phase fluctuations in the received echoes are due to the hydrometeors in V6.

1	6	change to read: “Note that the round-trip differential phase”

249			so as not to be ambiguous with used in earlier chapters (e.g., Eq.2.2) change, everywhere on this page and in Fig.8.15, , the apparent canting angle.

255	1	2	At the end of this line add a footnote mark and enter the footnote: Recent data from a disdrometer show as much as a factor of 3 error.

275	Eq. (8.77)	to make clear the distinction between the Doppler spectrumand the spectrum of terminal velocities, insert in front for Eq. (8.77) =. After this equation write “where v is the Doppler velocity, and it is assumed w and are spatially uniform, and that the resolution volume is…. for all diameters. However, it can be shown that Eq. (8.77) also applies to spatially non uniform and w. In that case, is replaced by , w by , and by .	Furthermore, must be free…..”

378	4	4	change to read “…can lift small scattering objects (e.g., insects, vegetation, debris, etc.) mostly confined…”

307			After Eq. (9.9) insert: Although the arc distance  on the earth’s surface (Fig.9.7), calculated using Eqs. (9.8) and (9.9), has a form different than s given by Eq. (2.28c), it can be shown, because r << ae , that the two expressions give the same result to first order in r/ae.

391	0	2	it should be noted that the correlation scale ρ0 is not the same as the integral scale ρI which is defined as

For the correlation function given by Eq. (10.19), ρ0 is related to ρI  as

398			Section 10.2.1: we introduce the variableΦv(K), the spatial spectrum of point velocities, in Eq.(10.46) but define it in the paragraph following its introduction (i.e., in Eq. 10.48). We should label (10.48) as (10.46), and place it before Eq.(10.46) and label it as (10.47). Other adjustments should be made to correct equation numbers; these should be few.

399	0	1	add the parenthetical phrase “(i.e., spectra of radial velocities along a radial and transverse to the direction of advecting turbulence)”

403	1	6	for a fuller explanation of the steps in Section 10.2.2, and using notation consistent with that used in earlier chapters (i.e., using E[x] instead of <x> to denote the expected value), we offer the following revision of Section 10.2.2:

In this section we define the relationship between the variance of radial velocities at a point and the expected spectrum width measured by radar (Rogers and Tripp, 1964).  Let the variance of the radial velocity v(r, t) at a point be.  This variance is the sum of the variance at all velocity scales and is defined by the equation,

(10.55)

where Ev[x] indicates an expectation, or an average over a complete ensemble of velocity fields, all having the same statistical properties. Assume that steady wind is not present, the radar beam is fixed, and hydrometeors do not oscillate or wobble and are perfect tracers of the wind. In this case, turbulence is the only mechanism contributing to spectral broadening, and it is a random variable having zero mean (i.e., Ev[v(r, t)] = 0).

The second central moment,, of the Doppler spectrum associated with turbulence can be obtained from Eq. (5.51).  Although Eq. (5.51) was derived under the assumption that v(r, t) is steady, this equation can be applied to the time varying wind produced by turbulence.  But then  would be a time varying quantity because v(r, t) is now a time dependent variable.  Replacing subscript ‘v’ with ‘t’ for pure turbulence, we obtain,

(10.56)

where  is the expected instantaneous second central moment of the Doppler spectrum associated with turbulence.  Although  is a function of r0, the location of the V6, we have omitted r0 to simplify notation; nevertheless, the argument r0 is implicit in . The overbar denotes a spatial average weighted by the normalized function  where

is a combination of reflectivity η(r)  and antenna pattern weights. Because the focus in this section is on the time changing velocity field we assume η(r) to be time independent.

Note that  is the expected instantaneous second central moment of the Doppler spectrum. In this case, however, the expectation, Eξ[x], is over ensembles of scatterer configurations ξ (Doviak and Zrnic, 1993, p.108) each having the same velocity field, whereas the expectation in Eq. (10.55) is taken over ensembles of velocity fields. The, expectationsEξ[x] can be made, at least in principle, over ensembles ofξ while v(r,tn)is frozen. Different scatterer configurations can be obtained by reshuffling scatterer locations. Moreover, although we can in principle freeze the velocity field, scatterers can have differential motion that results in a changing scatterer configuration. This in turn results in changes in the weather signal, and thus fluctuations of the estimates  of . The circumflex ^ denotes the estimate made from weather signal samples obtained during a dwell-time, Td, and ‘tn’ denotes the nth dwell-time, the short time span of duration MTs, typically less than 1s. Even though estimate variance associated with different configurations of scatterers is removed by a ξ average, we retain the circumflex ‘^’ to emphasize the estimated value in this section pertains to one member of the ensemble of velocity fields at the nth dwell-time.

The time dependence of  (i.e., of; henceforth the expectation over ξ will not be explicitly shown) is also due to changes of turbulence on scales large compared to V6 dimensions. Large scales of turbulence are said to create shear across V6. In this case large scale turbulence contributes a time varying shear component to  that can cause significant fluctuations of .  Usually we are not interested in the detailed time dependence of , but in its statistical properties such as its mean or expected value, (i.e., ), its auto-correlation function, etc. In this section we show how  is related to the energy density E of turbulence.

The  weighted radial velocity v(r,tn)is defined as the first moment  of the Doppler spectrum estimated from weather signal samples collected during Td. The variance of is, by definition, given by

.			(10.57a)

For pure turbulence,  = 0 and thus

.					(10.57b)

Because (10.56) applies to one member of an ensemble of velocity fields, we express (10.56) as

Where the diacritical simply emphasizes that is an estimate for one Td which essentially samples the velocity field. By taking the velocity ensemble average of this equation and substituting Eq. (10.57b) into it, we obtain, after commuting ensemble and spatial averages (i.e., ),

.			          (10.58a)

The weighted spatial average of Ev[v2(r,tn)] is, by definition,

(10.58b)

The derivation leading to Eq. (10.58) does not require turbulence to be statistically stationary, homogeneous, or isotropic. That is, Eq. (10.58a) relates the expected value of the second central moment of measured Doppler spectra (i.e., measured estimated with short dwell-times), and the variance of the mean Doppler velocity, to the  weighted spatial average of the expected value of the second central moment of the radial component of turbulence at each point r and tn. This is in agreement with results of Rogers and Tripp (1964).

These results apply to estimates of the Doppler velocity and second central moments made with any dwell-time. If longer dwell-times are used,  increases because velocity components associated with large scale turbulence have time to evolve within the resolution volume and be adequately sampled. On the other hand, the variance  of the mean Doppler velocity decreases as dwell time increases, and it vanishes in the limit of an infinite dwell time.  In this limit,  solely measures the spatial average of the weighted distribution of turbulence at each point.

10.2.2.1 Estimate variance due to changes in scatterer configuration

It should be noted that the variance, , does not include the variance associated with the statistical uncertainty due to changing scatterer configurations. Nevertheless, variance (e.g., that discussed in Chapter 6) associated with the weather signal fluctuations due to changes in the scatterer configuration can be significant, and it needs to be included in any rigorous analysis of radar measurements of turbulence.

For example, in addition to the variance of v(r,tn) due to the time changing velocity field, we have additional variance associated with the random location of scatterers (i.e., the time dependence of the true v(r,tn) differs from the time dependentv(r,tn)R estimated with radar). Here v(r,tn) is the weighted velocity irrespective of the scatterer configuration.

Even if v(r,tn) was a constant independent of time, the radar estimates  would be fluctuating due to the fact that scatterers move continuously to new locations for the same velocity field. That is, there is an evolving configuration of scatterers, and each configuration of scatterers produces a different weather signal sample from which v(r,tn) is estimated. In general, time fluctuations of estimates are due to both a changing velocity field and a changing configuration of scatterers.

To illustrate, assume a constant wind that carries scatterers along range arcs.  In this case, the radial velocity field v(r,tn) = 0.  Nevertheless, radar estimates of  v(r,tn) are time varying and random; this is so because the scatterers’ configuration within V6 continually changes as new scatterers enter V6, and others leave it. That is, the In-phase, I, and Quadrature, Q, components of the weather signal are still Gaussian distributed random variables as shown in Fig. 4.4a. In other words, although the mean or expected Doppler velocity is zero, the time sequence of the I, Q, samples will randomly move in the I, Q plane, and Doppler velocity estimates made with a small number of samples (e.g., two) can have non zero values.

The changes of I, Q from sample pair to sample pair can be relatively small if the sample pair spacing is short compared to the correlation time of the weather signals, and if the intra-pulse spacing Ts  <<. The weather signal correlation time, equal to the time required to flush V6 with new scatterers, is not necessarily equal to the correlation time of the velocity field; in our simple illustration the correlation time of the velocity field is infinite.  The non zero velocity estimates, calculated from pairs of I, Q samples, are uncorrelated if the pair spacing is longer than.  Only with a long time average will these velocity estimates average to 0.

These arguments, applied to the radar estimates ofv(r,tn), can also be applied to show that the ξ expectation of the radar estimates  {i.e., } equals .

10.2.2.2 Homogeneous turbulence

If turbulence is homogeneous over the region where the weighting functions contribute significantly (i.e., turbulence is locally homogeneous), Eq. (10.58b) shows that  = , the “point-measured variance” (Frisch and Clifford, 1974; the following paragraph will clarify what is meant by “point-measured variance”).  The radial component of the turbulent energy density at a point is,, where γ  is the air mass density. Using Eq. (10.58a), and noting that  = , we can then relate Er  to radar measurements as

.			(10.59)

If turbulence is isotropic, the total turbulence energy density E = 3Er.

Eq. (10.59), establishes a relation between the radial component of the “point-measured” turbulent energy density and the second central moment of the Doppler spectrum associated with turbulence, but it requires turbulence to be locally homogeneous although not isotropic or stationary.  Therefore, the “point” under discussion is, in reality, a collection of points over the entire resolution volume wherein turbulence is assumed to have the same statistical properties at each point. Section 10.2.2.3 presents results for the case where turbulence is inhomogeneous.

Eq. (10.59) demonstrates that the energy density of the radial component of “point-measured” homogeneous turbulence can be calculated from the sum of the expected value of the second central moment, , and the variance, , of the mean Doppler velocity estimates. It also shows how that energy is partitioned between large and small scales of turbulence;  is principally due to large scale turbulence whereas  is principally due to small scale turbulence.

If the radial component of turbulence has a -5/3rds power law vs wavenumberK=2πΛ for all wavelengths Λof the spectrum of turbulence, and if the dimensions of V6 are the same in all directions (i.e., ; Section 5.3), it can be shown that turbulence from all , the characteristic size of V6, contributes only about 20% to. Thus, although some portion of  is due to small scales, most of its contribution comes from turbulence on wavelengths large compared to Lc; this appears at variance with previously published interpretations. For example, if the weighting function is uniform, as stipulated by Rogers and Tripp (1964), across V6 having dimensions LxLxL, only 36% of  is due to turbulence from all Λ≤L. This contradicts Rogers and Tripp (1964) statement that   “receives spectral contributions mainly from the wavelengths shorter than the dimensions of V6”.

Large scale (i.e., large compared to V6 dimensions) turbulence shifts the Doppler spectrum along the velocity axis so that the single spectrum mean Doppler velocity changes from one spectrum to the next.  Thus to estimate  we need to average the second central moments calculated about each of the fluctuating means. For stationary and/or globally homogeneous turbulence, expectations can be obtained from averages over time and/or space (i.e., at different r0 locations).

10.2.2.3 Inhomogeneous turbulence

It is not necessary to assume turbulence is homogeneous (as we did in arriving at Eq. 10.59) to obtain a relation between the point variance of the radial component of turbulence and radar measurements. If turbulence is not homogeneous,  is still the variance at a point r, but  is the  weighted spatial average wind variance at each point. Then the expression for the point variance must be written as,

.

This is exactly the same form as Eq. (10.59), but we now have an overbar on . This simply means that radar can only measure the  weighted spatial average of turbulence at each and every point.

As stated earlier (Section 10.2.2.1), the variance σv2 does not include the variance associated with the statistical uncertainty of the estimates of due to weather signal fluctuations (i.e., the variance associated with changes in the scatterers’ configuration). The variance associated with the statistical uncertainty of the estimates must be subtracted from the measured variance in order to obtain ; window biases, typically associated with the measurements of  (Doviak and Zrnic, 1984, Fig. 6.8; Melnikov and Doviak, 2002), must also be taken into account.

430			Change the title of Section 11.3 to: “Solution for Fields Scattered by Refractive Index Perturbations” and everywhere else “irregularities” appear, change this word to “perturbations”

1	5-6	Change “spectrum of turbulence” to “spatial spectrum of refractive index perturbations”

2	2	after “links,” insert “and because enhanced is typically confined to thin layers,”

Fig.11.2	change figure caption to read: “….of the scattering geometry in a vertical plane containing T, R, and the spheroidal…….spheroid, ato and aro are unit vectors at the origin defined by the intersection of the transmitting and receiving beam axes. Vectors are indicated…..”

Fig.11.2		A comment on bistatic radar measurements:

Fig.11.2 would be more applicable to dual-Doppler radar remote sensing (i.e., measuring the Doppler shift of echoes received at T and at R) to map winds if the width of a narrow transmitting beam (e.g., 1o) was shown. For wind mapping the receiver beam is often much broader and fixed to receive bistatic echoes from anywhere the transmitting beam intersects scatterers. The location of the scatterers along the transmitting beam is determined by the arrival time of signals received at R. Thus, if the refractive perturbations are spatially uniform, the bistatic scattering volume (; shaded area) would be bounded by the volume common to the spheroidal shell of thickness  and the transmitter beam. In this case the elemental scattering volume dV would be confined to VB where the origin of is at the intersection of the transmitter’s beam axis with the spheroidal surface centered within the shell of thicknessdefined by Eq. (11.23). Thus there is a continuous distribution of  along the transmitted beam, and the location of each  is determined by the time signals are received at R.

431	2	15, 16	change to read: “…The range rr0 from the receiver to  is” and in Eq. (11.22) change rr to rr0.

18	change to “where is not the receiving beam’s elevation angle, but the elevation angle to , and 2d is the ….”

20	modify to: “…pulse is radiated, and it determines the location of the spheroidal surface centered within the shell of thickness. Thus ts determines the locations of spheroidal shells along the transmitter beam. Obviously……”

443 Section 11.4.3	to differentiate the commonly known Bragg scatter associated with steady or deterministic perturbations from that Bragg scatter associated with random perturbations, we introduce the term “Stochastic Bragg Scatter” by replacing the second sentence of this section with:

“Perturbations in atmospheric refractive index are caused by temperature and humidity fluctuations; thus the perturbation in n is a random variable having a spectrum of scales.  Although there is a spectrum of spatial scales, only those at about the Bragg wavelength ΛB = λ/[2sin(θs/2)] contribute significantly to the backscattered power.  Because scatter is from spatial fluctuations in refractive index, the scattering mechanism is herein defined as Stochastic Bragg Scatter (SBS). Because there are temporal fluctuations as well, the scattered power is also a random variable and its properties are related to the statistical properties of the scattering medium. In this section we relate the expected…..(return to the 3rd sentence in the text)”

459 Eq. (11.124)	this equation assumes that the beam width is given by Eq. (3.2b).  A more general form is

4		at the end of this paragraph, “...in this section.”, add: “Under far field conditions the beamwidth part of the “resolution volume weighting” term in Eq.(11.122) does not contribute significantly to the integral, but beamwidth and range resolution do contribute to the backscattered power because they multiply the integral in Eq.(11.122).”

460	0	2	add the following sentence at the end of the line:

ρh is the outer scale of the refractive index irregularities, but condition (11.124) applies to the transverse correlation lengths of the Bragg scatterers. Thus, the conclusion reached in this paragraph applies if the Bragg scatterer’s correlation length equals the outer scale.

1	49	the following comments offer an alternative and hopefully better explanation: The angular width (i.e., ≈ λ/ρB) of the diffraction pattern for each Bragg scatterer narrows as the correlation length of the Bragg scatterer increases, but reaches a limit λ/2rF, where rF is the first Fresnel zone radius.  Because the diffraction pattern’s main lobe is centered about the forward scatter direction (i.e., horizontally extended Bragg scatterers displaced horizontally from above the radar, have most significant radiation in a forward scatter direction such that the angle of reflection is equal to the angle of incidence), only those Bragg scatterers within λzo/ρB can contribute significantly to the receiver.  The Fresnel term accounts for this diminished contribution from Bragg scatterers.

461	0	11	insert after “...in space.”: “This is a consequence of the greater importance of the Fresnel term relative to the resolution volume weighting term (i.e., in Eq.11.122) along the transverse directions.”

478	0	7	rewrite the sentence beginning with “Then g is related….” as: “If the radiation pattern is well defined by a Gaussian function, g (now the directivity, Section 3.1.2) is related to  as…”

Appendix A.2		for clarification and consistency with notations used in Section 2.2.3, make the following revisions:

1	2	“….relate scatterer height hs to radar ….”. Also change h to hs everywhere in Appendix A.2 if it refers to the height of the scatterer (e.g., the upper limit of integration in Eqs. (A.8) and (A.9).

2	1-10	In a time interval (range-time; Section 3.4.2) a transmitted microwave pulse travels along a path to a scatterer at range l (in chapter 2 we use ‘r’ for true range) and returns as a microwave pulse (echo) of considerably reduced power. Range-time is the time delay, after the time a microwave pulse is transmitted, that an echo is received (Section 3.4.2). If the refractive index n is uniform along the path, the path is straight and the two-way distance is. But if n is a function of distance l along the path, the path is curved and the differential length dl at any point along the path is

Radar meteorologists typically assign an apparent range ra to the scatterer assuming the pulse travels at the constant speed c of light in free space. Thus the apparent differential range is

, and thus

is the relation between the true path length l (i.e., true range r) of the scatterer and the radar measured apparent range ra.

3	1	for consistency and clarity, modify to “……deduce that , so that…

8	modify to read: “…substitute hs for the upper integration limit in Eq. (A.5a) to obtain…”

513	3	4	rewrite as: “…independent of all others because the shell is assumed to be many wavelengths thick and scatterers are randomly placed in the shell.

547 	Index		add: “Antenna; far field, 435-436, 459”

548	Index		add: “Bright band, pp. 256, 268”

554	Index		add “Melting layer, pp. 225, 255”

556	Index		for the entry “Radome losses” add page 43.

These errata are periodically updated and posted on NSSL’s website at www.nssl.noaa.gov. In the “Quick Links” box select “Publications” to open the page to select “Recent Books” to find the book and listed Errata for the 3rd and 4th printings. Following these errata are supplements that clarify or extend the book text. The authors appreciate comments from users of the book; these comments have led to updated errata and supplements that help to keep the book correct and current.

The Dover Edition is a copy of the 1st and 2nd printings (not the 3rd and 4th printings as stated in the preface to the Dover edition; Oops!) of the 1993 edition of the Academic Press publication. Errata to the 1st and 2nd printings are also at the same NSSL website. Readers of the Dover edition need to refer to both errata to have all corrections.

Fang, M., and R. J. Doviak, 2008: Coupled Contributions in the Doppler Radar Spectrum Width Equation. J. Atmos. Oceanic Technol., 25, 2245-2258.

The 1998 report and its errata are available at NSSL’s website at www.nssl.noaa.gov under publications/reports.

This 2017 Report “A Memorandum on Comparisons of Weather and Aircraft Surveillance Radar Requirements to Determine Key Features for a 10-cm MPAR and SENSR (A Basis for a Cylindrical Polarimetric Phase Array Radar Design) is available at NSSL’s website at www.nssl.noaa.gov under publications/reports.

Harris, F. J., 1978: On the use of Windows for Harmonic Analysis with the Discrete Fourier Transform. Proc. IEEE, Vol. 66, No.1, January.

Ivić I. R., C. Curtis, and S. M. Torres, 2013: Radial-based Noise Power Estimation for Weather Radars, J. Atmos.Oceanic Technol., 30, 2737-2753. http://journals.ametsoc.org/doi/abs/10.1175/JTECH-D-13-00008.1

Although N can be calculated with negligible variance, the expected value can depend, especially for high performance radars such as the WSR-88D, on beam direction due to external noise sources factors such as the sun, earth temperature, precipitation, etc.

Melnikov, M. et al., 2011:  “Mapping Bragg Scatter with a Polarimetric WSR-88D”, Jo. Atmos. & Oceanic Tech., 28, Oct. 1273-1285.

NSSL 2017 Report “A Memorandum on Comparisons of Weather and Aircraft Surveillance Radar Requirements to Determine Key Features for a 10-cm MPAR and SENSR (A Basis for a Cylindrical Polarimetric Phase Array Radar Design) is available at NSSL’s website at www.nssl.noaa.gov under publications/reports.

Mishchenko, M.I., 2000: Calculation of the amplitude matrix for a non-spherical particle in a fixed orientation.  Appl. Opt., 39, 1026-1031.

? Ryzhkov, A., P. Zhang, J. Krause, T. Schuur, R. Palmer, and D. S. Zrnic, 2011: Simultaneous  measurements of precipitation using S-band and C-band polarimetric radars. Weather Radar and Hydrology International Symposium, WRaH 2011, Exeter, UK.

Doviak, R. J., V. Bringi, A. Ryzhkov, A. Zahrai, D. S. Zrnic', 2000: Considerations for Polarimetric Upgrades to Operational WSR-88D Radars. Jo. Atmos. and Oceanic Tech., March, 257-278.

Doviak, R. J., D. S. Zrnic, 1998: Polarimetric Upgrades to Improve Rainfall Measurements. NSSL Report, April,      110pp.

Zrnić, D. S., R. J. Doviak, G. Zhang, and A. Ryzhkov, 2010: Bias in Differential Reflectivity due to Cross Coupling through the Radiation Patterns of Polarimetric Weather Radars. Jo. Atmos. And Oceanic Tech., 27,1624-1637.

There are many reasons why the STSR mode is used by the network of WSR-88s, one of which is the cost and reliability of the components required to alternately switch the polarization of the transmitted RF pulse. Other practical reasons are given in Section 3.3 of the 2012 online NSSL Report: Issues and challenges for polarimetric measurement of weather with agile beam phased-array radar, 117 pp.